STEREO IMPACT SWEA LVPS COLD START REWORK INSTRUCTIONS

The SWEA load (Actel) has an in-rush transient at cold temperatures that the current supply design cannot accommodate, causing it to fail to start up.  In order to fix this problem, the supply current limit delay must be stretched significantly.  To do this, we will add a 4.7uF capacitor in parallel with C40.

- Solder 2” long #26 bus wire to a CWR06NC475KC capacitor from STEREO flight sock.  Place tubing over the bare wire.

- Capacitor Mfg, LDC___________________

- Wire traceability_______________________

- Tubing traceability_____________________

- Stake capacitor to the bottom of the SWEA board at the location shown in the figure below.  Avoid vias and traces when locating the part.  Remove the conformal coat in the area to get a good bond.  Use Uralane 5753 A/B

- Uralane batch & expiration________________

- Mix ratio___________________

- Cure at +60C for at least 4 hours.  Record duration_____________

- Solder the wires from the capacitor to the pads indicated in the figure (C40).  J-hook the wires to the existing leads.  Verify that the top-side solder and confomal coat for C40 is still OK.

- Stake the wires to the board with Uralane 5753 A/B.

- Uralane batch & expiration________________

- Mix Ratio__________________

- Touch up the conformal coat using Uralane 5750 A/B

- Uralane batch & expiration________________

- Mix Ratio__________________

- Cure at +60C for 12 hours.  Record duration________________

- COMPLETED BY ______________________________	DATE ______________________

- APPROVED BY: __________________________	DATE: __________________

+